AX-88 RS232 Codes

Code				Function

20 10 01 01 A0		OUT 1 MUTE

20 10 01 00 FE		OUT 1 UNMUTE

20 12 01 00 B1		OUT 1 VOLUME 0%

20 12 01 19 B0		OUT 1 VOLUME 25%

20 12 01 32 B3		OUT 1 VOLUME 50%

20 12 01 4B D7		OUT 1 VOLUME 75%

20 12 01 64 B5		OUT 1 VOLUME 100%

20 12 01 81 63		OUT 1 VOLUME UP

20 12 01 82 81		OUT 1 VOLUME DOWN

20 14 01 00 60		OUT 1 BASS 0%

20 14 01 19 61		OUT 1 BASS 25%

20 14 01 32 62		OUT 1 BASS 50%

20 14 01 4B 06		OUT 1 BASS 75%

20 14 01 64 64		OUT 1 BASS 100%

20 14 01 81 B2		OUT 1 BASS UP

20 14 01 82 50		OUT 1 BASS DOWN

20 16 01 00 2F		OUT 1 TREBLE 0%

20 16 01 19 2E		OUT 1 TREBLE 25%

20 16 01 32 2D		OUT 1 TREBLE 50%

20 16 01 4B 49		OUT 1 TREBLE 75%

20 16 01 64 2B		OUT 1 TREBLE 100%

20 16 01 81 FD		OUT 1 TREBLE UP

20 16 01 82 1F		OUT 1 TREBLE DOWN

20 18 01 00 DB		OUT 1 SUBWOOFER 0%

20 18 01 19 DA		OUT 1 SUBWOOFER 25%

20 18 01 32 D9		OUT 1 SUBWOOFER 50%

20 18 01 4B BD		OUT 1 SUBWOOFER 75%

20 18 01 64 DF		OUT 1 SUBWOOFER 100%

20 18 01 81 09		OUT 1 SUBWOOFER UP

20 18 01 82 EB		OUT 1 SUBWOOFER DOWN

20 10 02 01 F5		OUT 2 MUTE

20 10 02 00 AB		OUT 2 UNMUTE

20 12 02 00 E4		OUT 2 VOLUME 0%

20 12 02 19 E5		OUT 2 VOLUME 25%

20 12 02 32 E6		OUT 2 VOLUME 50%

20 12 02 4B 82		OUT 2 VOLUME 75%

20 12 02 64 E0		OUT 2 VOLUME 100%

20 12 02 81 36		OUT 2 VOLUME UP

20 12 02 82 D4		OUT 2 VOLUME DOWN

20 14 02 00 35		OUT 2 BASS 0%

20 14 02 19 34		OUT 2 BASS 25%

20 14 02 32 37		OUT 2 BASS 50%

20 14 02 4B 53		OUT 2 BASS 75%

20 14 02 64 31		OUT 2 BASS 100%

20 14 02 81 E7		OUT 2 BASS UP

20 14 02 82 05		OUT 2 BASS DOWN

20 16 02 00 7A		OUT 2 TREBLE 0%

20 16 02 19 7B		OUT 2 TREBLE 25%

20 16 02 32 78		OUT 2 TREBLE 50%

20 16 02 4B 1C		OUT 2 TREBLE 75%

20 16 02 64 7E		OUT 2 TREBLE 100%

20 16 02 81 A8		OUT 2 TREBLE UP

20 16 02 82 4A		OUT 2 TREBLE DOWN

20 18 02 00 8E		OUT 2 SUBWOOFER 0%

20 18 02 19 8F		OUT 2 SUBWOOFER 25%

20 18 02 32 8C		OUT 2 SUBWOOFER 50%

20 18 02 4B E8		OUT 2 SUBWOOFER 75%

20 18 02 64 8A		OUT 2 SUBWOOFER 100%

20 18 02 81 5C		OUT 2 SUBWOOFER UP

20 18 02 82 BE		OUT 2 SUBWOOFER DOWN

20 10 03 01 31		OUT 3 MUTE

20 10 03 00 6F		OUT 3 UNMUTE

20 12 03 00 20		OUT 3 VOLUME 0%

20 12 03 19 21		OUT 3 VOLUME 25%

20 12 03 32 22		OUT 3 VOLUME 50%

20 12 03 4B 46		OUT 3 VOLUME 75%

20 12 03 64 24		OUT 3 VOLUME 100%

20 12 03 81 F2		OUT 3 VOLUME UP

20 12 03 82 10		OUT 3 VOLUME DOWN

20 14 03 00 F1		OUT 3 BASS 0%

20 14 03 19 F0		OUT 3 BASS 25%

20 14 03 32 F3		OUT 3 BASS 50%

20 14 03 4B 97		OUT 3 BASS 75%

20 14 03 64 F5		OUT 3 BASS 100%

20 14 03 81 23		OUT 3 BASS UP

20 14 03 82 C1		OUT 3 BASS DOWN

20 16 03 00 BE		OUT 3 TREBLE 0%

20 16 03 19 BF		OUT 3 TREBLE 25%

20 16 03 32 BC		OUT 3 TREBLE 50%

20 16 03 4B D8		OUT 3 TREBLE 75%

20 16 03 64 BA		OUT 3 TREBLE 100%

20 16 03 81 6C		OUT 3 TREBLE UP

20 16 03 82 8E		OUT 3 TREBLE DOWN

20 18 03 00 4A		OUT 3 SUBWOOFER 0%

20 18 03 19 4B		OUT 3 SUBWOOFER 25%

20 18 03 32 48		OUT 3 SUBWOOFER 50%

20 18 03 4B 2C		OUT 3 SUBWOOFER 75%

20 18 03 64 4E		OUT 3 SUBWOOFER 100%

20 18 03 81 98		OUT 3 SUBWOOFER UP

20 18 03 82 7A		OUT 3 SUBWOOFER DOWN

20 10 04 01 5F		OUT 4 MUTE

20 10 04 00 01		OUT 4 UNMUTE

20 12 04 00 4E		OUT 4 VOLUME 0%

20 12 04 19 4F		OUT 4 VOLUME 25%

20 12 04 32 4C		OUT 4 VOLUME 50%

20 12 04 4B 28		OUT 4 VOLUME 75%

20 12 04 64 4A		OUT 4 VOLUME 100%

20 12 04 81 9C		OUT 4 VOLUME UP

20 12 04 82 7E		OUT 4 VOLUME DOWN

20 14 04 00 9F		OUT 4 BASS 0%

20 14 04 19 9E		OUT 4 BASS 25%

20 14 04 32 9D		OUT 4 BASS 50%

20 14 04 4B F9		OUT 4 BASS 75%

20 14 04 64 9B		OUT 4 BASS 100%

20 14 04 81 4D		OUT 4 BASS UP

20 14 04 82 AF		OUT 4 BASS DOWN

20 16 04 00 D0		OUT 4 TREBLE 0%

20 16 04 19 D1		OUT 4 TREBLE 25%

20 16 04 32 D2		OUT 4 TREBLE 50%

20 16 04 4B B6		OUT 4 TREBLE 75%

20 16 04 64 D4		OUT 4 TREBLE 100%

20 16 04 81 02		OUT 4 TREBLE UP

20 16 04 82 E0		OUT 4 TREBLE DOWN

20 18 04 00 24		OUT 4 SUBWOOFER 0%

20 18 04 19 25		OUT 4 SUBWOOFER 25%

20 18 04 32 26		OUT 4 SUBWOOFER 50%

20 18 04 4B 42		OUT 4 SUBWOOFER 75%

20 18 04 64 20		OUT 4 SUBWOOFER 100%

20 18 04 81 F6		OUT 4 SUBWOOFER UP

20 18 04 82 14		OUT 4 SUBWOOFER DOWN

20 10 05 01 9B		OUT 5 MUTE

20 10 05 00 C5		OUT 5 UNMUTE

20 12 05 00 8A		OUT 5 VOLUME 0%

20 12 05 19 8B		OUT 5 VOLUME 25%

20 12 05 32 88		OUT 5 VOLUME 50%

20 12 05 4B EC		OUT 5 VOLUME 75%

20 12 05 64 8E		OUT 5 VOLUME 100%

20 12 05 81 58		OUT 5 VOLUME UP

20 12 05 82 BA		OUT 5 VOLUME DOWN

20 14 05 00 5B		OUT 5 BASS 0%

20 14 05 19 5A		OUT 5 BASS 25%

20 14 05 32 59		OUT 5 BASS 50%

20 14 05 4B 3D		OUT 5 BASS 75%

20 14 05 64 5F		OUT 5 BASS 100%

20 14 05 81 89		OUT 5 BASS UP

20 14 05 82 6B		OUT 5 BASS DOWN

20 16 05 00 14		OUT 5 TREBLE 0%

20 16 05 19 15		OUT 5 TREBLE 25%

20 16 05 32 16		OUT 5 TREBLE 50%

20 16 05 4B 72		OUT 5 TREBLE 75%

20 16 05 64 10		OUT 5 TREBLE 100%

20 16 05 81 C6		OUT 5 TREBLE UP

20 16 05 82 24		OUT 5 TREBLE DOWN

20 18 05 00 E0		OUT 5 SUBWOOFER 0%

20 18 05 19 E1		OUT 5 SUBWOOFER 25%

20 18 05 32 E2		OUT 5 SUBWOOFER 50%

20 18 05 4B 86		OUT 5 SUBWOOFER 75%

20 18 05 64 E4		OUT 5 SUBWOOFER 100%

20 18 05 81 32		OUT 5 SUBWOOFER UP

20 18 05 82 D0		OUT 5 SUBWOOFER DOWN

20 10 06 01 CE		OUT 6 MUTE

20 10 06 00 90		OUT 6 UNMUTE

20 12 06 00 DF		OUT 6 VOLUME 0%

20 12 06 19 DE		OUT 6 VOLUME 25%

20 12 06 32 DD		OUT 6 VOLUME 50%

20 12 06 4B B9		OUT 6 VOLUME 75%

20 12 06 64 DB		OUT 6 VOLUME 100%

20 12 06 81 0D		OUT 6 VOLUME UP

20 12 06 82 EF		OUT 6 VOLUME DOWN

20 14 06 00 0E		OUT 6 BASS 0%

20 14 06 19 0F		OUT 6 BASS 25%

20 14 06 32 0C		OUT 6 BASS 50%

20 14 06 4B 68		OUT 6 BASS 75%

20 14 06 64 0A		OUT 6 BASS 100%

20 14 06 81 DC		OUT 6 BASS UP

20 14 06 82 3E		OUT 6 BASS DOWN

20 16 06 00 41		OUT 6 TREBLE 0%

20 16 06 19 40		OUT 6 TREBLE 25%

20 16 06 32 43		OUT 6 TREBLE 50%

20 16 06 4B 27		OUT 6 TREBLE 75%

20 16 06 64 45		OUT 6 TREBLE 100%

20 16 06 81 93		OUT 6 TREBLE UP

20 16 06 82 71		OUT 6 TREBLE DOWN

20 18 06 00 B5		OUT 6 SUBWOOFER 0%

20 18 06 19 B4		OUT 6 SUBWOOFER 25%

20 18 06 32 B7		OUT 6 SUBWOOFER 50%

20 18 06 4B D3		OUT 6 SUBWOOFER 75%

20 18 06 64 B1		OUT 6 SUBWOOFER 100%

20 18 06 81 67		OUT 6 SUBWOOFER UP

20 18 06 82 85		OUT 6 SUBWOOFER DOWN

20 10 07 01 0A		OUT 7 MUTE

20 10 07 00 54		OUT 7 UNMUTE

20 12 07 00 1B		OUT 7 VOLUME 0%

20 12 07 19 1A		OUT 7 VOLUME 25%

20 12 07 32 19		OUT 7 VOLUME 50%

20 12 07 4B 7D		OUT 7 VOLUME 75%

20 12 07 64 1F		OUT 7 VOLUME 100%

20 12 07 81 C9		OUT 7 VOLUME UP

20 12 07 82 2B		OUT 7 VOLUME DOWN

20 14 07 00 CA		OUT 7 BASS 0%

20 14 07 19 CB		OUT 7 BASS 25%

20 14 07 32 C8		OUT 7 BASS 50%

20 14 07 4B AC		OUT 7 BASS 75%

20 14 07 64 CE		OUT 7 BASS 100%

20 14 07 81 18		OUT 7 BASS UP

20 14 07 82 FA		OUT 7 BASS DOWN

20 16 07 00 85		OUT 7 TREBLE 0%

20 16 07 19 84		OUT 7 TREBLE 25%

20 16 07 32 87		OUT 7 TREBLE 50%

20 16 07 4B E3		OUT 7 TREBLE 75%

20 16 07 64 81		OUT 7 TREBLE 100%

20 16 07 81 57		OUT 7 TREBLE UP

20 16 07 82 B5		OUT 7 TREBLE DOWN

20 18 07 00 71		OUT 7 SUBWOOFER 0%

20 18 07 19 70		OUT 7 SUBWOOFER 25%

20 18 07 32 73		OUT 7 SUBWOOFER 50%

20 18 07 4B 17		OUT 7 SUBWOOFER 75%

20 18 07 64 75		OUT 7 SUBWOOFER 100%

20 18 07 81 A3		OUT 7 SUBWOOFER UP

20 18 07 82 41		OUT 7 SUBWOOFER DOWN

20 10 08 01 12		OUT 8 MUTE

20 10 08 00 4C		OUT 8 UNMUTE

20 12 08 00 03		OUT 8 VOLUME 0%

20 12 08 19 02		OUT 8 VOLUME 25%

20 12 08 32 01		OUT 8 VOLUME 50%

20 12 08 4B 65		OUT 8 VOLUME 75%

20 12 08 64 07		OUT 8 VOLUME 100%

20 12 08 81 D1		OUT 8 VOLUME UP

20 12 08 82 33		OUT 8 VOLUME DOWN

20 14 08 00 D2		OUT 8 BASS 0%

20 14 08 19 D3		OUT 8 BASS 25%

20 14 08 32 D0		OUT 8 BASS 50%

20 14 08 4B B4		OUT 8 BASS 75%

20 14 08 64 D6		OUT 8 BASS 100%

20 14 08 81 00		OUT 8 BASS UP

20 14 08 82 E2		OUT 8 BASS DOWN

20 16 08 00 9D		OUT 8 TREBLE 0%

20 16 08 19 9C		OUT 8 TREBLE 25%

20 16 08 32 9F		OUT 8 TREBLE 50%

20 16 08 4B FB		OUT 8 TREBLE 75%

20 16 08 64 99		OUT 8 TREBLE 100%

20 16 08 81 4F		OUT 8 TREBLE UP

20 16 08 82 AD		OUT 8 TREBLE DOWN

20 18 08 00 69		OUT 8 SUBWOOFER 0%

20 18 08 19 68		OUT 8 SUBWOOFER 25%

20 18 08 32 6B		OUT 8 SUBWOOFER 50%

20 18 08 4B 0F		OUT 8 SUBWOOFER 75%

20 18 08 64 6D		OUT 8 SUBWOOFER 100%

20 18 08 81 BB		OUT 8 SUBWOOFER UP

20 18 08 82 59		OUT 8 SUBWOOFER DOWN

20 10 00 01 64		OUT ALL MUTE

20 10 00 00 3A		OUT ALL UNMUTE

20 12 00 00 75		OUT ALL VOLUME 0%

20 12 00 19 74		OUT ALL VOLUME 25%

20 12 00 32 77		OUT ALL VOLUME 50%

20 12 00 4B 13		OUT ALL VOLUME 75%

20 12 00 64 71		OUT ALL VOLUME 100%

20 12 00 81 A7		OUT ALL VOLUME UP

20 12 00 82 45		OUT ALL VOLUME DOWN

20 14 00 00 A4		OUT ALL BASS 0%

20 14 00 19 A5		OUT ALL BASS 25%

20 14 00 32 A6		OUT ALL BASS 50%

20 14 00 4B C2		OUT ALL BASS 75%

20 14 00 64 A0		OUT ALL BASS 100%

20 14 00 81 76		OUT ALL BASS UP

20 14 00 82 94		OUT ALL BASS DOWN

20 16 00 00 EB		OUT ALL TREBLE 0%

20 16 00 19 EA		OUT ALL TREBLE 25%

20 16 00 32 E9		OUT ALL TREBLE 50%

20 16 00 4B 8D		OUT ALL TREBLE 75%

20 16 00 64 EF		OUT ALL TREBLE 100%

20 16 00 81 39		OUT ALL TREBLE UP

20 16 00 82 DB		OUT ALL TREBLE DOWN

20 18 00 00 1F		OUT ALL SUBWOOFER 0%

20 18 00 19 1E		OUT ALL SUBWOOFER 25%

20 18 00 32 1D		OUT ALL SUBWOOFER 50%

20 18 00 4B 79		OUT ALL SUBWOOFER 75%

20 18 00 64 1B		OUT ALL SUBWOOFER 100%

20 18 00 81 CD		OUT ALL SUBWOOFER UP

20 18 00 82 2F		OUT ALL SUBWOOFER DOWN

20 06 00 01 FF		RECALL MEMORY 1	(Audio Output Settings)

20 06 00 02 1D		RECALL MEMORY 2	(Audio Output Settings)

20 06 00 03 43		RECALL MEMORY 3	(Audio Output Settings)

20 06 00 04 C0		RECALL MEMORY 4	(Audio Output Settings)

20 06 00 05 9E		RECALL MEMORY 5	(Audio Output Settings)

20 06 00 06 7C		RECALL MEMORY 6	(Audio Output Settings)

20 06 00 07 22		RECALL MEMORY 7	(Audio Output Settings)

20 06 00 08 63		RECALL MEMORY 8	(Audio Output Settings)

AX-88 does not support EDID

20 20 00 00 E4		EDID FIXED

20 20 00 01 BA		EDID FROM OUTPUT 1

AX-88 does not support video output

20 05 00 01 1B		RECALL MEMORY 1	(Video Output Settings)

20 05 00 02 F9		RECALL MEMORY 2	(Video Output Settings)

20 05 00 03 A7		RECALL MEMORY 3	(Video Output Settings)

20 05 00 04 24		RECALL MEMORY 4	(Video Output Settings)

20 05 00 05 7A		RECALL MEMORY 5	(Video Output Settings)

20 05 00 06 98		RECALL MEMORY 6	(Video Output Settings)

20 05 00 07 C6		RECALL MEMORY 7	(Video Output Settings)

20 05 00 08 87		RECALL MEMORY 8	(Video Output Settings)

Switch Audio for AX-88

| Switch Audio | Command Code |  |  |  |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Output 1 | 20 | 02 | 01 | 00 | FB | Output 1 disconnect |  |
|  | 20 | 02 | 01 | 01 | A5 | Output 1 select Input 1 |  |
|  | 20 | 02 | 01 | 02 | 47 | Output 1 select Input 2 |  |
|  | 20 | 02 | 01 | 03 | 19 | Output 1 select Input 3 |  |
|  | 20 | 02 | 01 | 04 | 9A | Output 1 select Input 4 |  |
|  | 20 | 02 | 01 | 05 | C4 | Output 1 select Input 5 |  |
|  | 20 | 02 | 01 | 06 | 26 | Output 1 select Input 6 |  |
|  | 20 | 02 | 01 | 07 | 78 | Output 1 select Input 7 |  |
|  | 20 | 02 | 01 | 08 | 39 | Output 1 select Input 8 |  |
| Output 2 | 20 | 02 | 02 | 00 | AE | Output 2 disconnect |  |
|  | 20 | 02 | 02 | 01 | F0 | Output 2 select Input 1 |  |
|  | 20 | 02 | 02 | 02 | 12 | Output 2 select Input 2 |  |
|  | 20 | 02 | 02 | 03 | 4C | Output 2 select Input 3 |  |
|  | 20 | 02 | 02 | 04 | CF | Output 2 select Input 4 |  |
|  | 20 | 02 | 02 | 05 | 91 | Output 2 select Input 5 |  |
|  | 20 | 02 | 02 | 06 | 73 | Output 2 select Input 6 |  |
|  | 20 | 02 | 02 | 07 | 2D | Output 2 select Input 7 |  |
|  | 20 | 02 | 02 | 08 | 6C | Output 2 select Input 8 |  |
| Output 3 | 20 | 02 | 03 | 00 | 6A | Output 3 disconnect |  |
|  | 20 | 02 | 03 | 01 | 34 | Output 3 select Input 1 |  |
|  | 20 | 02 | 03 | 02 | D6 | Output 3 select Input 2 |  |
|  | 20 | 02 | 03 | 03 | 88 | Output 3 select Input 3 |  |
|  | 20 | 02 | 03 | 04 | 0B | Output 3 select Input 4 |  |
|  | 20 | 02 | 03 | 05 | 55 | Output 3 select Input 5 |  |
|  | 20 | 02 | 03 | 06 | B7 | Output 3 select Input 6 |  |
|  | 20 | 02 | 03 | 07 | E9 | Output 3 select Input 7 |  |
|  | 20 | 02 | 03 | 08 | A8 | Output 3 select Input 8 |  |
| Output 4 | 20 | 02 | 04 | 00 | 04 | Output 4 disconnect |  |
|  | 20 | 02 | 04 | 01 | 5A | Output 4 select Input 1 |  |
|  | 20 | 02 | 04 | 02 | B8 | Output 4 select Input 2 |  |
|  | 20 | 02 | 04 | 03 | E6 | Output 4 select Input 3 |  |
|  | 20 | 02 | 04 | 04 | 65 | Output 4 select Input 4 |  |
|  | 20 | 02 | 04 | 05 | 3B | Output 4 select Input 5 |  |
|  | 20 | 02 | 04 | 06 | D9 | Output 4 select Input 6 |  |
|  | 20 | 02 | 04 | 07 | 87 | Output 4 select Input 7 |  |
|  | 20 | 02 | 04 | 08 | C6 | Output 4 select Input 8 |  |
| Output 5 | 20 | 02 | 05 | 00 | C0 | Output 5 disconnect |  |
|  | 20 | 02 | 05 | 01 | 9E | Output 5 select Input 1 |  |
|  | 20 | 02 | 05 | 02 | 7C | Output 5 select Input 2 |  |
|  | 20 | 02 | 05 | 03 | 22 | Output 5 select Input 3 |  |
|  | 20 | 02 | 05 | 04 | A1 | Output 5 select Input 4 |  |
|  | 20 | 02 | 05 | 05 | FF | Output 5 select Input 5 |  |
|  | 20 | 02 | 05 | 06 | 1D | Output 5 select Input 6 |  |
|  | 20 | 02 | 05 | 07 | 43 | Output 5 select Input 7 |  |
|  | 20 | 02 | 05 | 08 | 02 | Output 5 select Input 8 |  |
| Output 6 | 20 | 02 | 06 | 00 | 95 | Output 6 disconnect |  |
|  | 20 | 02 | 06 | 01 | CB | Output 6 select Input 1 |  |
|  | 20 | 02 | 06 | 02 | 29 | Output 6 select Input 2 |  |
|  | 20 | 02 | 06 | 03 | 77 | Output 6 select Input 3 |  |
|  | 20 | 02 | 06 | 04 | F4 | Output 6 select Input 4 |  |
|  | 20 | 02 | 06 | 05 | AA | Output 6 select Input 5 |  |
|  | 20 | 02 | 06 | 06 | 48 | Output 6 select Input 6 |  |
|  | 20 | 02 | 06 | 07 | 16 | Output 6 select Input 7 |  |
|  | 20 | 02 | 06 | 08 | 57 | Output 6 select Input 8 |  |
| Output 7 | 20 | 02 | 07 | 00 | 51 | Output 7 disconnect |  |
|  | 20 | 02 | 07 | 01 | 0F | Output 7 select Input 1 |  |
|  | 20 | 02 | 07 | 02 | ED | Output 7 select Input 2 |  |
|  | 20 | 02 | 07 | 03 | B3 | Output 7 select Input 3 |  |
|  | 20 | 02 | 07 | 04 | 30 | Output 7 select Input 4 |  |
|  | 20 | 02 | 07 | 05 | 6E | Output 7 select Input 5 |  |
|  | 20 | 02 | 07 | 06 | 8C | Output 7 select Input 6 |  |
|  | 20 | 02 | 07 | 07 | D2 | Output 7 select Input 7 |  |
|  | 20 | 02 | 07 | 08 | 93 | Output 7 select Input 8 |  |
| Output 8 | 20 | 02 | 08 | 00 | 49 | Output 8 disconnect |  |
|  | 20 | 02 | 08 | 01 | 17 | Output 8 select Input 1 |  |
|  | 20 | 02 | 08 | 02 | F5 | Output 8 select Input 2 |  |
|  | 20 | 02 | 08 | 03 | AB | Output 8 select Input 3 |  |
|  | 20 | 02 | 08 | 04 | 28 | Output 8 select Input 4 |  |
|  | 20 | 02 | 08 | 05 | 76 | Output 8 select Input 5 |  |
|  | 20 | 02 | 08 | 06 | 94 | Output 8 select Input 6 |  |
|  | 20 | 02 | 08 | 07 | CA | Output 8 select Input 7 |  |
|  | 20 | 02 | 08 | 08 | 8B | Output 8 select Input 8 |  |
| All Outputs | 20 | 02 | 00 | 00 | 3F | All Outputs disconnect |  |
|  | 20 | 02 | 00 | 01 | 61 | All Outputs select Input 1 |  |
|  | 20 | 02 | 00 | 02 | 83 | All Outputs select Input 2 |  |
|  | 20 | 02 | 00 | 03 | DD | All Outputs select Input 3 |  |
|  | 20 | 02 | 00 | 04 | 5E | All Outputs select Input 4 |  |
|  | 20 | 02 | 00 | 05 | 00 | All Outputs select Input 5 |  |
|  | 20 | 02 | 00 | 06 | E2 | All Outputs select Input 6 |  |
|  | 20 | 02 | 00 | 07 | BC | All Outputs select Input 7 |  |
|  | 20 | 02 | 00 | 08 | FD | All Outputs select Input 8 |  |

20 04 00 01 B0		Audio Settings Save to Memory 1

20 04 00 02 52		Audio Settings Save to Memory 2

20 04 00 03 0C		Audio Settings Save to Memory 3

20 04 00 04 8F		Audio Settings Save to Memory 4

20 04 00 05 D1		Audio Settings Save to Memory 5

20 04 00 06 33		Audio Settings Save to Memory 6

20 04 00 07 6D		Audio Settings Save to Memory 7

20 04 00 08 2C		Audio Settings Save to Memory 8

20 06 00 01 FF		Audio Settings Load from Memory 1

20 06 00 02 1D		Audio Settings Load from Memory 2

20 06 00 03 43		Audio Settings Load from Memory 3

20 06 00 04 C0		Audio Settings Load from Memory 4

20 06 00 05 9E		Audio Settings Load from Memory 5

20 06 00 06 7C		Audio Settings Load from Memory 6

20 06 00 07 22		Audio Settings Load from Memory 7

20 06 00 08 63		Audio Settings Load from Memory 8

Request Audio Status

20 11 00 00 2D		Request All Outputs Mute Status.

20 13 00 00 DE		Request All Outputs Volume (%).

20 15 00 00 0F		Request All Outputs Bass (%).

20 17 00 00 40		Request All Outputs Treble (%).

20 19 00 00 B4		Request All Outputs Subwoofer (%).